NdFeB Magnet Recycling Using Hydrogen Decrepitation

Hydrogen Decrepitation (HD) Based Rare Earth Element Recovery

E-waste NdFeB Magnets  →  Dismantling & Cleaning  →  Hydrogen Decrepitation  →  Fine Powder  →  Acid Leaching  →  Separation & Precipitation  →  Nd / Pr / Dy / Tb + Fe Recovery

# 1. E-Waste NdFeB Magnets

End-of-life NdFeB permanent magnets are collected from e-waste and subjected to dismantling and cleaning before further processing.

# 2. Dismantling & Cleaning

Magnets are separated from the waste stream and cleaned to remove unwanted attachments and contaminants before Hydrogen Decrepitation.

# 3. Hydrogen Decrepitation (HD)

Hydrogen Decrepitation converts NdFeB magnets into a friable powder through hydrogen absorption, hydride formation, lattice expansion and cracking.

## 3.1 Hydrogen Absorption

Hydrogen (H₂) diffuses into the NdFeB magnet and reacts with the alloy phases.

## 3.2 Hydride Formation

Hydrogen reacts with the Nd-rich phase and promotes hydride formation. The associated volume expansion generates internal stresses.

Chemical Reaction Nd₂Fe₁₄B + xH₂ → Nd₂Fe₁₄B·Hₓ

## 3.3 Cracking / Decrepitation

H₂ diffusion  →  hydride formation  →  lattice / volume expansion  →  internal stress  →  cracking  →  fine NdFeB powder

# 4. Fine Powder Generation

The brittle magnet breaks down into fine particles, making subsequent hydrometallurgical treatment easier and reducing the need for conventional high-temperature melting.

# 5. Acid Leaching

The HD powder can be treated using a suitable acid — HCl, H₂SO₄ or HNO₃. Acid selection and operating conditions require experimental optimization.

Leachate  →  Fe removal / recovery  →  REE separation  →  precipitation

# 6. Rare Earth Element Separation

The rare-earth-rich stream is processed to recover the principal NdFeB-associated rare earth elements: Nd (Neodymium), Pr (Praseodymium), Dy (Dysprosium) and Tb (Terbium). Other elements depend on the magnet composition.

# 7. Iron Recovery

Iron is separated as a side stream rather than being treated only as waste. Depending on the downstream process, Fe may be recovered as an iron salt or iron oxide.

Leaching solution  ↙ REEs → Nd / Pr / Dy / Tb        ↘ Fe → Fe salt / Fe oxide

# 8. Overall Material Flow

E-waste NdFeB magnet → Dismantling & cleaning → H₂ absorption → Hydride formation → Expansion → Cracking → Fine powder → Acid leaching → Fe / REE separation → Nd / Pr / Dy / Tb recovery → Fe recovery → Safe waste treatment

# 9. HD vs HDDR

HD (Hydrogen Decrepitation) focuses on hydrogen-assisted embrittlement and powder generation. HDDR (Hydrogenation–Disproportionation–Desorption–Recombination) is a more advanced treatment that can modify microstructure and magnetic properties. HDDR should only be included as an actual process step if it is experimentally performed in the project.

# 10. Key Process Parameters

Parameters should be reported as experimental conditions or validated process ranges, not assumed fixed values.

| Parameter | Suggested Range |
| --- | --- |
| H₂ pressure | 2 – 10 bar |
| Temperature | 300 – 500 °C |
| Reaction time | 1 – 10 h |
| H₂ purity | ≥ 99.9% |
| Particle size (before HD) | 1 – 5 mm |
| Powder size (after HD) | ~10 – 100 μm |

# 11. Comparison with Conventional Pyrometallurgy

| Aspect | HD + Hydrometallurgy |
| --- | --- |
| Processing temperature | Lower-temperature route |
| Melting requirement | Avoids melting in the powder-generation stage |
| Downstream handling | Produces a leachable powder directly |
| REE recovery | Potential for selective REE recovery |
| Energy demand | Comparatively lower thermal energy input |

| Aspect | Conventional Pyrometallurgy |
| --- | --- |
| Processing temperature | Very high temperature |
| Melting requirement | Requires melting |
| Downstream handling | Metal separation only after melting |
| REE recovery | Less selective, bulk metal recovery |
| Energy demand | More energy intensive |

# 12. Advantages

✅  Lower thermal requirement

✅  High REE recovery potential

✅  Direct powder generation

✅  Reduced requirement for melting

✅  Compatible with hydrometallurgical separation

# 13. Challenges

⚠  H₂ safety requirements

⚠  High-purity hydrogen requirement

⚠  Acid consumption

⚠  Wastewater treatment

⚠  Process optimization requirements

# 14. Environmental & Circular-Economy Considerations

Potential indicators for evaluation include energy consumption, acid consumption, H₂ consumption, CO₂ emissions, REE recovery percentage, Fe recovery percentage and waste generation. Experimental data should be used before assigning numerical performance claims.

# 15. Example Material Balance

Input NdFeB magnets → Cleaned magnets → HD powder → REE-rich leachate + Fe-rich stream → Nd / Pr / Dy / Tb concentrates + Fe product

Example material balance — experimental values required. Actual quantities must be obtained experimentally; no numerical values are assumed here.